Forced charging operation steps

Step  1 Open the maintenance cover of battery ,  find the positive pole + and negative pole - of the battery. Check voltage by multimeter ,For example 60V.

Negative pole -

Positive pole +

Step 2

Adjust the wire position.  Loosen 2 nuts and remove the wire from position 1  to position 2,then fix the nuts

position 2

position 1

Step 3

Click up-left side of the charger screen , then find Dubug mode setting,Input Password 228958.

Then input voltage 80V, Current 5A, click the confirm button. the charger starts working.

Step 4

Stop charging when the voltage is more than 80 V. Adjust the wire to its initial position.(from position 2  to position 1),Then start charging to 100% SOC.

position 1

position 2